SAFETY DATA SHEETS

According to the UN GHS revision 11

Version: 1.0

Creation Date: October 12, 2025

Revision Date: October 12, 2025

SECTION 1: Identification

1.1GHS Product identifier

| Product name | 4-CPA |
| --- | --- |

1.2Other means of identification

| Product number | - |
| --- | --- |
| Other names | marks4-cpa; parachlorophenoxyacetic acid; Sure-Set |

1.3Recommended use of the chemical and restrictions on use

| Identified uses | Industrial and scientific research use. |
| --- | --- |
| Uses advised against | no data available |

1.4Supplier's details

| Company | chemicalbook |
| --- | --- |
| Address | Renmin East Road 3786,Shanyang District, Jiaozuo City, Henan, China |
| Telephone | 86-391-2161601 |

1.5Emergency phone number

| Emergency phone number | 13608638553 |
| --- | --- |
| Service hours | Monday to Friday, 9am-5pm (Standard time zone: UTC/GMT +8 hours). |

SECTION 2: Hazard identification

2.1Classification of the substance or mixture

Acute toxicity - Category 4, Oral

2.2GHS label elements, including precautionary statements

| Pictogram(s) |  |
| --- | --- |
| Signal word | Warning |
| Hazard statement(s) | H302 Harmful if swallowed |
| Precautionary statement(s) |  |
| Prevention | P264 Wash ... thoroughly after handling. P270 Do not eat, drink or smoke when using this product. |
| Response | P301+P317 IF SWALLOWED: Get medical help. P330 Rinse mouth. |
| Storage | none |
| Disposal | P501 Dispose of contents/container to an appropriate treatment and disposal facility in accordance with applicable laws and regulations, and product characteristics at time of disposal. |

2.3Other hazards which do not result in classification

no data available

SECTION 3: Composition/information on ingredients

3.1Substances

| Chemical name | Common names and synonyms | CAS number | EC number | Concentration |
| --- | --- | --- | --- | --- |
| 4-CPA | 4-CPA | 122-88-3 | 204-581-3 | 100% |

SECTION 4: First-aid measures

4.1Description of necessary first-aid measures

If inhaled

Move the victim into fresh air. If breathing is difficult, give oxygen. If not breathing, give artificial respiration and consult a doctor immediately. Do not use mouth to mouth resuscitation if the victim ingested or inhaled the chemical.

Following skin contact

Take off contaminated clothing immediately. Wash off with soap and plenty of water. Consult a doctor.

Following eye contact

Rinse with pure water for at least 15 minutes. Consult a doctor.

Following ingestion

Rinse mouth with water. Do not induce vomiting. Never give anything by mouth to an unconscious person. Call a doctor or Poison Control Center immediately.

4.2Most important symptoms/effects, acute and delayed

no data available

4.3Indication of immediate medical attention and special treatment needed, if necessary

If any symptoms of illness occur during or following inhalation of spray, remove victim from contact with the material for at least 2 to 3 days. Allow subsequent contact with chlorophenoxy compounds only if effective respiratory protection is practiced. Chlorophenoxy compounds

SECTION 5: Fire-fighting measures

5.1Suitable extinguishing media

Use dry chemical, carbon dioxide or alcohol-resistant foam.

5.2Specific hazards arising from the chemical

no data available

5.3Special protective actions for fire-fighters

Wear self-contained breathing apparatus for firefighting if necessary.

SECTION 6: Accidental release measures

6.1Personal precautions, protective equipment and emergency procedures

Avoid dust formation. Avoid breathing mist, gas or vapours.Avoid contacting with skin and eye. Use personal protective equipment.Wear chemical impermeable gloves. Ensure adequate ventilation.Remove all sources of ignition. Evacuate personnel to safe areas.Keep people away from and upwind of spill/leak.

6.2Environmental precautions

Prevent further spillage or leakage if it is safe to do so. Do not let the chemical enter drains. Discharge into the environment must be avoided.

6.3Methods and materials for containment and cleaning up

If material is released or spilled, flush with water .

SECTION 7: Handling and storage

7.1Precautions for safe handling

Handling in a well ventilated place. Wear suitable protective clothing. Avoid contact with skin and eyes. Avoid formation of dust and aerosols. Use non-sparking tools. Prevent fire caused by electrostatic discharge steam.

7.2Conditions for safe storage, including any incompatibilities

Store the container tightly closed in a dry, cool and well-ventilated place. Store apart from foodstuff containers or incompatible materials.

SECTION 8: Exposure controls/personal protection

8.1Control parameters

Occupational Exposure limit values

no data available

Biological limit values

no data available

8.2Appropriate engineering controls

Ensure adequate ventilation. Handle in accordance with good industrial hygiene and safety practice. Set up emergency exits and the risk-elimination area.

8.3Individual protection measures, such as personal protective equipment (PPE)

Eye/face protection

Wear tightly fitting safety goggles with side-shields conforming to EN 166(EU) or NIOSH (US).

Skin protection

Wear fire/flame resistant and impervious clothing. Handle with gloves. Gloves must be inspected prior to use. Wash and dry hands. The selected protective gloves have to satisfy the specifications of EU Directive 89/686/EEC and the standard EN 374 derived from it.

Respiratory protection

If the exposure limits are exceeded, irritation or other symptoms are experienced, use a full-face respirator.

Thermal hazards

no data available

SECTION 9: Physical and chemical properties and safety characteristics

| Physical state | white to light beige powder |
| --- | --- |
| Colour | Prisms or needles from water |
| Odour | no data available |
| Melting point/freezing point | 157-159°C |
| Boiling point or initial boiling point and boiling range | 315.2°C at 760mmHg |
| Flammability | no data available |
| Lower and upper explosion limit/flammability limit | no data available |
| Flash point | no data available |
| Auto-ignition temperature | no data available |
| Decomposition temperature | no data available |
| pH | no data available |
| Kinematic viscosity | no data available |
| Solubility | In water, 957 mg/L at 25 deg C |
| Partition coefficient n-octanol/water | log Kow = 2.25 |
| Vapour pressure | 0.000188mmHg at 25°C |
| Density and/or relative density | 1.366 g/cm3 |
| Relative vapour density | no data available |
| Particle characteristics | no data available |

SECTION 10: Stability and reactivity

10.1Reactivity

no data available

10.2Chemical stability

It is stable at elevated temperatures (54 deg C) and on exposure to metals, but degrades (30% loss in 24 hours) on exposure to sunlight.

10.3Possibility of hazardous reactions

no data available

10.4Conditions to avoid

no data available

10.5Incompatible materials

no data available

10.6Hazardous decomposition products

When heated to decomposition it emits toxic fumes of /hydrogen chloride/.

SECTION 11: Toxicological information

Acute toxicity

- Oral: LD50 Rat oral 850 mg/kg

- Inhalation: no data available

- Dermal: no data available

Skin corrosion/irritation

no data available

Serious eye damage/irritation

no data available

Respiratory or skin sensitization

no data available

Germ cell mutagenicity

no data available

Carcinogenicity

no data available

Reproductive toxicity

no data available

STOT-single exposure

no data available

STOT-repeated exposure

no data available

Aspiration hazard

no data available

SECTION 12: Ecological information

12.1Toxicity

- Toxicity to fish: LC50 Lepomis macrochirus (Bluegill) >180 ppm/96 hr; static /diethanolamine salt; 2% AI formulated product

- Toxicity to daphnia and other aquatic invertebrates: no data available

- Toxicity to algae: no data available

- Toxicity to microorganisms: no data available

12.2Persistence and degradability

AEROBIC: The time required for 80% breakdown of 4-chlorophenoxyacetic acid in soil was 12.1 days in fresh soil and 2.5 days in adapted soil(1). The number of days for complete removal in Honeoye and Dunkirk soils was 94 and 11 days, respectively(2). 4-Chlorophenoxyacetic acid had a 31% theoretical biological oxygen demand in a 5 day test using an acclimated mixed microbial culture(3). A pilot plant sewage treatment plant provided 32-58% removal using an activated sludge inocula, residence time not provided(4). 4-Chlorophenoxyacetic acid degraded in mixed soil and sewage inocula(5).

12.3Bioaccumulative potential

An estimated BCF of 3 was calculated for 4-chlorophenoxyacetic acid(SRC), using a log Kow of 2.25(1) and a regression-derived equation(2). According to a classification scheme(3), this BCF suggests the potential for bioconcentration in aquatic organisms is low(SRC).

12.4Mobility in soil

The Rf value for 4-chlorophenoxyacetic acid ranged from 0.74 at pH 4.0 to 0.93 at pH 7 using a Rothansted silty clay loam(1). At pH 7.8-7.9, 4-chlorophenoxyacetic acid was found not to be significantly adsorbed to clay(2). Sorption of 4-chlorophenoxy acetic acid to clay is pH dependent and increases with decreasing ionization(3). Using a structure estimation method based on molecular connectivity indices(4), the Koc for 4-chlorophenoxy acetic acid can be estimated to be 18(SRC). According to a classification scheme(5), this estimated Koc value suggests that 4-chlorophenoxy acetic acid is expected to have very high mobility in soil(SRC). In addition, the pKa of 4-chlorophenoxy acetic acid is 3.56(6), indicating that this compound will primarily exist as an anion in the environment, and anions generally possess high mobility in soil(7).

12.5Other adverse effects

no data available

SECTION 13: Disposal considerations

13.1Disposal methods

Product

The material can be disposed of by removal to a licensed chemical destruction plant or by controlled incineration with flue gas scrubbing. Do not contaminate water, foodstuffs, feed or seed by storage or disposal. Do not discharge to sewer systems.

Contaminated packaging

Containers can be triply rinsed (or equivalent) and offered for recycling or reconditioning. Alternatively, the packaging can be punctured to make it unusable for other purposes and then be disposed of in a sanitary landfill. Controlled incineration with flue gas scrubbing is possible for combustible packaging materials.

SECTION 14: Transport information

14.1UN Number

| ADR/RID: Not dangerous goods. (For reference only, please check.) | IMDG: Not dangerous goods. (For reference only, please check.) | IATA: Not dangerous goods. (For reference only, please check.) |
| --- | --- | --- |

14.2UN Proper Shipping Name

| ADR/RID: Not dangerous goods. (For reference only, please check.) | IMDG: Not dangerous goods. (For reference only, please check.) | IATA: Not dangerous goods. (For reference only, please check.) |
| --- | --- | --- |

14.3Transport hazard class(es)

| ADR/RID: Not dangerous goods. (For reference only, please check.) | IMDG: Not dangerous goods. (For reference only, please check.) | IATA: Not dangerous goods. (For reference only, please check.) |
| --- | --- | --- |

14.4Packing group, if applicable

| ADR/RID: Not dangerous goods. (For reference only, please check.) | IMDG: Not dangerous goods. (For reference only, please check.) | IATA: Not dangerous goods. (For reference only, please check.) |
| --- | --- | --- |

14.5Environmental hazards

| ADR/RID: No | IMDG: No | IATA: No |
| --- | --- | --- |

14.6Special precautions for user

no data available

14.7Transport in bulk according to IMO instruments

no data available

SECTION 15: Regulatory information

15.1Safety, health and environmental regulations specific for the product in question

| Chemical name | Common names and synonyms | CAS number | EC number |
| --- | --- | --- | --- |
| 4-CPA | 4-CPA | 122-88-3 | 204-581-3 |
| European Inventory of Existing Commercial Chemical Substances (EINECS) |  |  | Listed. |
| EC Inventory |  |  | Listed. |
| United States Toxic Substances Control Act (TSCA) Inventory |  |  | Listed. |
| China Catalog of Hazardous chemicals 2015 |  |  | Not Listed. |
| New Zealand Inventory of Chemicals (NZIoC) |  |  | Listed. |
| Philippines Inventory of Chemicals and Chemical Substances (PICCS) |  |  | Not Listed. |
| Vietnam National Chemical Inventory |  |  | Listed. |
| Chinese Chemical Inventory of Existing Chemical Substances (China IECSC) |  |  | Listed. |
| Korea Existing Chemicals List (KECL) |  |  | Listed. |

SECTION 16: Other information

Information on revision

| Creation Date | October 12, 2025 |
| --- | --- |
| Revision Date | October 12, 2025 |

Abbreviations and acronyms

- CAS: Chemical Abstracts Service

- ADR: European Agreement concerning the International Carriage of Dangerous Goods by Road

- RID: Regulation concerning the International Carriage of Dangerous Goods by Rail

- IMDG: International Maritime Dangerous Goods

- IATA: International Air Transportation Association

- TWA: Time Weighted Average

- STEL: Short term exposure limit

- LC50: Lethal Concentration 50%

- LD50: Lethal Dose 50%

- EC50: Effective Concentration 50%

References

- IPCS - The International Chemical Safety Cards (ICSC), website: http://www.ilo.org/dyn/icsc/showcard.home

- HSDB - Hazardous Substances Data Bank, website: https://toxnet.nlm.nih.gov/newtoxnet/hsdb.htm

- IARC - International Agency for Research on Cancer, website: http://www.iarc.fr/

- eChemPortal - The Global Portal to Information on Chemical Substances by OECD, website: http://www.echemportal.org/echemportal/index?pageID=0&request_locale=en

- CAMEO Chemicals, website: http://cameochemicals.noaa.gov/search/simple

- ChemIDplus, website: http://chem.sis.nlm.nih.gov/chemidplus/chemidlite.jsp

- ERG - Emergency Response Guidebook by U.S. Department of Transportation, website: http://www.phmsa.dot.gov/hazmat/library/erg

- Germany GESTIS-database on hazard substance, website: http://www.dguv.de/ifa/gestis/gestis-stoffdatenbank/index-2.jsp

- ECHA - European Chemicals Agency, website: https://echa.europa.eu/